NITRAZEPAM

Nitrazepam IP/ BP

Description :

Nitrazepam occurs as white to yellow crystals

or crystalline powder. It is odorless.

It is freely soluble in acetic acid (100), soluble in acetone and in chloroform, slightly soluble in methanol, in ethanol

(95)

and in ethanol

(99.5)

, very slightly soluble in

diethylether

, and practically insoluble in water.

o

Melting point: about 227 C (with decomposition).

Identiˆcation

(1) To 3 mL of a solution of Nitrazepam in methanol (1 in 500) add 0.1 mL of

sodium hydroxide TS: a yellow color is produced.

(2) To 0.02 g of Nitrazepam add 15 mL of dilute hydrochloric acid, boil for 5 minutes, cool, and ˆlter: the

ˆltrate responds to the

Qualitative Tests <1.09> for primary aromatic amines.

(3) Neutralize 0.5 mL of the ˆltrate obtained in (2) with sodium hydroxide TS, add 2 mL of ninhydrin TS, and heat

on a water bath: a purple color is produced.

(4) Determine the absorption spectrum of a solution of Nitrazepam in ethanol

(99.5) (1 in 100,000)

as

directed under Ultraviolet-visible Spectrophotometry

<2.24>,

and compare the spectrum with the Reference

Spectrum: both spectra ex- hibit similar intensities of absorption at the same wavelengths.

Purity

:

(1) Clarity and color of solution—

Dissolve 0.10 g of Nitrazepam in 20 mL of acetone: the solution is clear and

pale yellow to light yellow in color.

(2) Heavy metals <1.07>—

Proceed with 1.0 g of Nitrazep- am according to Method 2, and perform the test.

Prepare the control solution with 2.0 mL of Standard Lead Solution (not more than 20 ppm).

(3) Arsenic <1.11

>—Prepare the test solution with 1.0 g of Nitrazepam according to Method 3, and perform

the test (not more than 2 ppm).

(4) Related substances—

Dissolve 0.25 g of Nitrazepam in a 10 mL of mixture of methanol and chloroform

(1:1), and use this solution as the sample solution. Pipet 1 mL of the sample solution, add a mixture of

methanol and chloroform (1:1) to make exactly 20 mL, pipet 2 mL of this solution, add a mixture of methanol

and chloroform (1:1) to make exactly 50 mL, and use this solution as the standard solution. Per- form the test

with these solutions as directed under Thin-lay- er

Chromatography <2.03>. Spot 10 mL

each of the sample so-

lution and standard solution on a plate of silica gel with ‰uorescent indicator for thin-layer chromatography.

Develop the plate with a mixture of nitromethane and ethyl acetate (

17:3) to a distance of about 10 cm, and

air-dry the plate. Ex- amine under ultraviolet light (main wavelength: 254 nm)

: the spots other than the principal

spot from the sample solution are not more intense than the spot from the standard solution.

PRODUCT CODE:TAJ API NTWAK 88765323

CAS No. 146-22-5