|  | UPDATED ON AUGUST 2009 |
| --- | --- |
| Taj Pharmaceuticals Chemical List Fine Chemicals |  |
|  | PRODUCT CODE- AMPHT2458 |
| 3-Aminoacetophenone. Name : 3-Aminoacetophenone Synonyms : 3-Amino acetophenone; m-Aminoacetophenone; 1-(3-Aminophenyl)ethanone, 3-Aminoacetophenone, 3-Amino acetophenone, m-Aminoacetophenone, 1-(3-Aminophenyl)ethanone, CAS No. : 99-03-6 Molecular Formula : C8H9NO Molecular Weight : 135.16 EINECS 202-722-3 Properties Melting point 95-98 ºC Boiling point 289-290 ºC. Production methods : by the Inter-nitrophenyl ethanone derived using iron reduction, response cans of water, stirring to add iron, slowly to the Industrial hydrochloride. Further stages between nitrobenzene ethanone. Heating return 6h. Cool to 30 ° C, plus hydrochloric acid reaction was strongly acidic liquid. Filtration and the filtrate added to the ammonia was weakly alkaline, precipitated solids. Rejection filter, filter cake for inter-aminoacetophenone. Material Safety Data Sheet Hazards Identification Potential Acute Health Effects: Hazardous in case of eye contact (irritant), of inhalation. Slightly hazardous in case of skin contact (irritant,permeator), of ingestion. Potential Chronic Health Effects: CARCINOGENIC EFFECTS: Not available. MUTAGENIC EFFECTS: Not available. TERATOGENIC EFFECTS: Not available. DEVELOPMENTAL TOXICITY: Not available. Repeated or prolonged exposure is not known to aggravate medical condition. First Aid Measures. Eye Contact: Check for and remove any contact lenses. Immediately flush eyes with running water for at least 15 minutes,keeping eyelids open. Get medical attention. Finish by rinsing thoroughly with running water to avoid a possibleinfection. Skin Contact: Wash with soap and water. Cover the irritated skin with an emollient. Get medical attention if irritation develops. Serious Skin Contact: Not available. Inhalation: If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention. Serious Inhalation: Not available. Ingestion: Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. If large quantities of this material are swallowed, call a physician immediately. Loosen tight clothing such as a collar, tie, belt or waistband. Serious Ingestion: Not available. Handling and Storage Precautions: Keep away from heat. Keep away from sources of ignition. Empty containers pose a fire risk, evaporate the residue under a fume hood. Ground all equipment containing material. Do not ingest. Do not breathe dust. Avoid contact with eyes. Wear suitable protective clothing. In case of insufficient ventilation, wear suitable respiratory equipment. If ingested, seek medical advice immediately and show the container or the label. Keep away from incompatibles such as acids. Storage: Keep container tightly closed. Keep container in a cool, well-ventilated area. Note: These API/ chemicals are designated as those that are used in the manufacture of the controlled substances and are important to the manufacture of the substances. For any (Control Substance) products Import and Export *** subjected to your country government laws /control substance ACT. Note /Government Notification: N/A |
| This document plus the full buyer / prescribing information, prepared for health professionals can be found at: http://www.tajapi.com or by contacting the sponsor, Taj Pharmaceuticals Limited., at: 91 022 30601000. This leaflet was prepared by Taj Pharmaceuticals Limited, Mumbai (India). PRODUCT CODE- AMPHT2458 Last revised: 29 August 2009 |  |